|  | Application form (Annex 1) |  |  |
| --- | --- | --- | --- |
|  | DE-UZ 72 – Edition January 2026 | Printing and publication paper made primarily from recovered paper |  |

Company information

Druck- und Pressepapier überwiegend aus Altpapier

| Company name: |  |
| --- | --- |
| Full address: |  |

Contact person

| Name: |  |
| --- | --- |
| Function: |  |
| Phone number: |  |
| E-Mail-address: |  |

Product details

| Trade name of the product: |  |
| --- | --- |
| Production site: |  |

Mark which category the product belongs to

|  | 01 05 05 05 Standard newsprint paper |
| --- | --- |
|  | 01 05 10 05 Improved newsprint paper |
|  | 01 10 05 05 SC-A rotogravure paper |
|  | 01 10 10 05 SC-B rotogravure paper |
|  | 01 10 15 05 SC-A offset |
|  | 01 10 20 05 SC-B offset |
|  | 01 80 05 05 Wood-containing printing and writing paper, in rolls |
|  | 01 80 10 05 Wood-containing printing and writing paper, in formats |
|  | 01 85 05 06 Wood-containing printing and writing paper, coated on two sides, in rolls, LWC for rotogravure printing |
|  | 01 85 05 07 Wood-containing printing and writing paper, coated on two sides, in rolls, LWC for offset |
|  | 01 85 05 11 Wood-containing printing and writing paper, coated on two sides, in rolls, HWC for rotogravure printing |
|  | 01 85 05 12 Wood-containing printing and writing paper, coated on two sides, in rolls, HWC for offset |
|  | 01 85 10 05 Wood-containing printing and writing paper, coated on two sides, in formats, consumption, standard and special coating |

|  | The maximum weight of the printing and publication paper is 75 g/m2 |
| --- | --- |

| We hereby declare compliance with all applicable requirements according to Section 3 of the basic award criteria. |  |
| --- | --- |

3.1 Use of fibrous raw materials and grades of recovered paper

A minimum of 800 kg of post-consumer recovered paper per 1000 kg of new paper (air dry) must be used in the production of the paper.

Refer to DIN EN 643 for specifications about the different grades of recovered paper. The proportion of virgin fibres added to the product must not exceed a maximum of 250 kg per 1000 kg of new paper (air dry).

If virgin fibres are used, they must comply with the requirements in Paragraphs 3.16, 3.17 and 3.18.

At least 80 % of the recovered paper used for the product must be sourced from group 1 of the grades of recovered paper.

3.2 Diisopropylnaphthalene (DIPN)

The content of diisopropylnaphthalene (DIPN) in paper and cardboard should be kept as low as technically possible. It is thus generally not permitted to use the grades of recovered paper 2.05.00 ordinary sorted office paper, 2.05.01 sorted office paper, 2.06.00 ordinary sorted coloured letters, 2.06.01 sorted coloured letters and 5.09.00 “carbonless copy paper (NCR)”. Alternatively, grades of recovered paper containing DIPN (2.05.00, 2.05.01, 2.06.00, 2.06.01 and 5.09.00) may be used if an efficient technical system (e.g. deinking) exists that largely removes the DIPN from the fibre cycle and the DIPN content in the finished paper does not exceed a maximum of 50 mg/kg.

3.3 Colour developers from thermal paper

The use of recovered paper could transfer colour developers from thermal paper to the finished paper. Therefore, the content of bisphenol A (BPA), bisphenol S (BOS) and N-(p-toluolsulfonyl) N'-(3-(p-toluolsulfonyloxy)phenyl)urea in the finished paper must be determined in a cold water extract once a year.

Depending on the type of product, the content of BPA (CAS no. 80-05-7), BPS (CAS no. 80-09 1) and N-(p-toluolsulfonyl)-N'-(3-(p-toluolsulfonyloxy)phenyl)urea (CAS no. 232938-43-1) must be determined in a cold water extract prepared according to DIN EN 645 using liquid chromatography and UV/fluorescence detection or MS detection in accordance with CEN/TS 17497.

3.4 General exclusion of substances with certain properties

No substances may be added as colourants, coating materials, production aids and paper refining agents that have the following properties:

a) It is prohibited to add substances of very high concern (SVHC) that have been identified as being particularly alarming in accordance with Article 57, Paragraph 1 of Regulation (EC) No 1907/2006 (REACH) and added to the so-called “candidate list” according to Article 59, Paragraph 1 of the same regulation.

b) No substances may be added to the product that

- according to the criteria of Regulation (EC) No 1272/2008 (CLP) are classified with the following H Phrases named in Table 1 or which meet the criteria for such classification.

- are classified as carcinogenic, mutagenic or reprotoxic substances in the currently valid version of TRGS 905.

Table 1: H Phrases according to the CLP Regulation

| H Phrases according to the CLP Regulation | Wording |
| --- | --- |
| H340 | May cause genetic defects. |
| H341 | Suspected of causing genetic defects. |
| H350 | May cause cancer. |
| H350i | May cause cancer if inhaled. |
| H351 | Suspected of causing cancer. |
| H360F | May damage fertility. |
| H360D | May damage the unborn child. |
| H360FD | May damage fertility. May damage the unborn child. |
| H360Fd | May damage fertility. Suspected of damaging the unborn child. |
| H360Df | May damage the unborn child. Suspected of damaging fertility. |
| H361f | Suspected of damaging fertility. |
| H361d | Suspected of damaging the unborn child. |
| H361fd | Suspected of damaging fertility. Suspected of damaging the unborn child. |

3.5 Further requirements for production aids and paper refining agents

Only those production aids and paper refining agents that are listed in Recommendation XXXVI from the BfR (Federal Institute of Risk Assessment) for “Paper and board for food contact” (positive list) in sections B (production aids) and C (special paper refining agents) may be added to the product. The maximum quantities and concentrations stated in this list must be observed.

No production aids containing glyoxal may be used to manufacture the recycled paper.

3.6 Bleaching and complexing agents

The he recovered paper must be processed without the use of chlorine, halogenated bleaching agents and not readily biodegradable complexing agents such as e.g. ehylenediaminetetraacetic acid (EDTA) and diethylenetriaminepentaacetic acid (DTPA).

3.7 Biocides

In the production of the recycled paper, only those slimicides (substances in product type 12) and preservatives for fibres (substances in product type 9) that have been approved in accord ance with the Biocidal Products Regulation (EU) No. 528/2012 (EU list of approved active sub stances) or are still being examined as a notified existing active substance for the relevant type of biocides as part of the EU work programme for the systematic examination of all existing active substances may be used.

Accordingly, it is only permitted to use those biocidal products classified in product types 9 and 12 that have been explicitly approved for the desired application.

For a transitional period, biocidal products that contain notified existing active substances of product types 9 and 12 that are still being examined as part of the EU examination process can also be used without approval if they have been registered in accordance with the German ordinance on the notification of biocidal products pursuant to the German Chemicals Act (Biocide Notification Ordinance – ChemBiozidMeldeV) and can thus be made available on the market.

Until the approval requirements for the biocidal products containing notified existing active substances come into force, however, only those substances that are also listed in Recommendation XXXVI from the BfR are permitted.

In addition, the biocidal products used in the product must not contain any substances that have been considered as candidates for substitution according to Article 10 of the EU Biocidal Products Regulation 528/2012.

It is also possible that production aids and paper refining agents used for the production of the recycled paper contain biocidal products in product type 6 (preservatives for products during storage) that have been made available on the market. Residual content of these biocidal products will be accepted.

3.8 Optical brighteners

The use of optical brighteners is not permitted.

An exception applies for the production of SC and LWC paper (according to Appendix B). The following optical brighteners may be added to these products:

• C.I.220, benzenesulfonic acid, 2,2'-(1,2-ethenediyl) bis[5[4-[bis(2-hydroxyethyl) amino]-6 [(4-sulfophenyl)amino]- 1,3,5-triazin-2-yl]amino]-, tetra sodium salt (CAS no. 16470-24 9);

• C.I. 113 or C.I. 28 disodium salt 4,4'-bis[6-anilino-4-[bis(2-hydroxyethyl)amino]-1,3,5-tri azin-2-yl]amino]stilbene-2,2'-disulphonate; sulfonated stilbene derivatives may be used up to a maximum level of 0.3%;

• Tetrasodium 4,4'-{ethene-1,2-diylbis[(3-sulfonato-4,1-phenylene)imino{6-[bis(2-hydroxy ethyl)amino]-1,3,5-triazine-4,2-diyl}imino]}dibenzoate (CAS no. 32257-57-1) and isomeric mixtures of Tetrasodium 4,4'-{ethene-1,2-diylbis[(3-sulfonato-4,1-phenylene)-imino{6 [bis(2-hydroxyethyl)amino]-1,3,5-triazine-4,2-diyl}imino]}dibenzoate (CAS no. 32257-57 1), Tetrasodium 2,2'-{ethene-1,2-diylbis[(3-sulfonatobenzene-4,1-diyl)imino{6-[bis(2-hy droxyethyl)amino]-1,3,5-triazine-4,2-diyl}imino]}dibenzoate (CAS no. 158256-89-4) and Tetrasodium 2-({4-[bis(2-hydroxyethyl)amino]-6-[(4-{2-[4-({4-[bis(2-hydroxyethyl) amino]-6-[(4-carboxylatophenyl)amino]-1,3,5-triazin-2-yl}amino)-2-sulfonatophenyl]-eth enyl}-3-sulfonatophenyl)amino]-1,3,5-triazin-2-yl}amino)benzoate (CAS no. 1271742-13 2);

• C.I.397 (benzenesulfonic acid, 2,2‘-(1,2-ethenediyl)bis[5-amino-, reaction products with aniline, diethanolamine, ethanolamine and 2,4,6-trichloro-1,3,5-triazine, sodium salts, 2 (Dimethylamino) ethanol compounds (CAS no. 1627851-12-0).

3.9 Azo dyes and pigemts in colurants

No azo dyes or pigments may be added in colourants that can cleave into one of the amines stated in Regulation (EC) No. 1907/2006, Annex XVII, No. 43, Appendix 8 or 9, or in TRGS 614 (see Appendix C).

3.10 Mercury, lead, cadmium or chromium VI compounds in colourants

It is not permitted to add any colourants (pigments or dyes) containing mercury, lead, cadmium or chromium (VI) compounds as constituent ingredients.

3.11 Substitution of mineral oil-based additives and base oils

No mineral oil-based additives or base oils that contain aromatic hydrocarbons (with ≥ 10 carbon atoms) as a component may be added during the production of the recycled paper. In the case of aliphatic hydrocarbons, only those substances with a chain length of C10 to C20 may be used. Plant-based base oils designed for use as a foodstuff or as a material that comes into contact with foodstuffs are exempt from this requirement. Plant-based substitutes for mineral oil should be free of genetic engineering and sourced from sustainable cultivation.

- 3.12 Waste water

The following requirements apply with respect to waste water:

3.12.1 Direct discharge

In the production of the paper, including the relevant share of the pulp production (wood pulp or TMP), the emission limits for the waste water load that are listed by the EU Commission in the “Best Available Techniques (BAT) Reference Document for the Production of Pulp, Paper and Board PP BREF” must be complied with by direct dischargers. These limits have been implemented in Germany in Appendix 28 of the German Waste Water Ordinance. The values stated in Table 2 go above and beyond the statutory requirements in some cases.

Table 2: Maximum limits for the average annual emission parameters (waste water) in the paper production process

| Parameter | Maximum limit for waste water emissions (average annual value as a load or concentration) |
| --- | --- |
| Volumetric flow rate of waste water | 15 m3/Adt |
| COD | 3 kg/Adt |
| BSB5 | 0,15 kg/Adt or 25 mg/l |
| AOX | < 0,01 kg/Adt |
| Total N (inorganic + organic N) (TNb) | 0,07 kg/Adt or 15 mg/l |
| Total P | 0,008 kg/Adt or 1,2 mg/l |

Adt = air dried ton

TNb = total nitrogen bound. This parameter defines the total pollution of water by nitrogen compounds, which can appear in the form of e.g. ammonia, nitrites, nitrates or organic nitrogen compounds. This parameter must be deter mined using the method described in DIN EN ISO 20236.

3.12.2 Indirect discharge

Indirect dischargers must declare compliance with the emission limits for the parameters stated in Table 2 after treatment.

3.13 Emissions to air

Requirements are placed on the emissions to air in the production of recycled paper, including the relevant share of the pulp production (wood pulp or TMP). The emissions to air include both the emissions from the plants generating the steam needed for the production of the paper and also the emissions from the plants used to prepare the recovered paper as well as the paper machine. Emissions during production of the fibrous raw materials (DIP) also have to be taken into account. The limits listed here in Table 3 are based on the EU criteria for the award of the EU Ecolabel for graphic paper (Annex I) in the Official Journal of the European Union of 17 January 2019.

The applicant must determine the levels of the following pollutants in the emissions to air at the paper factory and should comply with the limits stated in Table 3 (measurement specifications, see Appendix D “Measurement of emissions to air in the production of paper”):

Table 3: Maximum limits for the average annual emission parameters (emissions to air) in the paper production process (average annual value (AAV) in kg/air dry tonne)

|  | Sulfur (S) as AAV | NOx as AAV |
| --- | --- | --- |
| Preparation of the recovered paper | 0,20 kg/t | 0,25 kg/t |
| Production of recycled paper | 0,30 kg/t | 0,50 kg/t |

3.14 Waste

To promote the avoidance of waste and in the spirit of a circular economy, the waste generated during the production process must be minimised. The following waste materials can be generated in the production of the paper: Fibrous sludge and sludge from the treatment of the process water. The following waste materials are also generated during the production of paper from recovered paper: Residues from the treatment of the recovered paper and deinking sludge. The remaining waste materials should be recycled as far as possible.

In the production of paper, the following waste material limits, including sludge from the treatment of the process water, given as an annual average figure as a dry mass, must not be exceeded:

Paper factory with deinking: 250 kg/t of product

Paper factory without deinking: 135 kg/t of product.

These limits were derived from the data collected for the BAT reference document for the paper and pulp industry (PP BREF, Section 6.2.1., Table 6.1).

3.15 Energy consumption and origin oft he electricity

The paper industry is one of the six most energy-intensive industries in Germany. The Blue Angel also sets incentives in its Basic Award Criteria for companies to further reduce the consumption of heat and electricity at the production sites. For the production of paper, including the relevant share of the pulp production (wood pulp or TMP), the limits for the consumption of electricity and process heat in the following table must not be exceeded as an annual average:

If deinked recovered paper (DIP) or pulp is purchased as a semi-finished product, the energy consumption for additional drying and transport processes must also be taken into account. The sum of the process heat and electrical power is permitted to exceed the limit by a total of 10%.

In addition, the applicant must state the energy mix used at the plant based on the type and origin of the energy. The consumed electricity should be sourced from renewable energies. The applicant must also state when he or she will stop using coal as an energy source, if this has not already occurred.

Table 4: Maximum limits for the consumption of process heat and electricity in the production of paper (annual average value in kWh/t)

|  | Process heat (including power-to-heat) in kWh/t | Electrical energy (excluding power-to-heat) in kWh/t |
| --- | --- | --- |
| Paper factory with deinking | 1.550 | 900 |
| Paper factory without deinking | 1.550 | 600 |
| Paper factory including pulp production | 1.550 | 2000 |

These limits were derived from the data collected for the BAT reference document for the paper and pulp industry (PP BREF, Section 6.2.1, Table 6.7) and for PTS Munich: Report No. 2 Use of energy saving techniques, for the revision of the BAT reference document for the pulp and paper industry 2009, Table 14, EU Ecolabel criteria for newsprint paper with pulp content).

3.16 Origin of the virgin fibres

It must be possible to verify the origin of the wood for the virgin fibres added to the product. The wood must be sourced from forests that are able to verify that they have been managed according to the guidelines for sustainable forestry. The relevant forestry business must work in accordance with a high ecological and social standard and be certified accordingly. The certification systems from the Forest Stewardship Council® (FSC), the Programme for the Endorsement of Forest Certification Schemes (PEFC) and the Naturland standard will be accepted.

Wood sourced from regional forestry businesses that remain close to nature also helps to avoid long transport routes, which have a negative ecological impact.

3.17 Requirements for the virgin fibres

Virgin fibres may only be added to products certified according to DE-UZ 72 up to the maximum amount permitted if they are produced without the use of any chlorine, halogenated bleaching agents and optical brighteners.

3.18 Requirements placed on the production of the virgin fibres

3.18.1 Waste water emissions in the production of the pulp

The applicant must determine the levels of the following chemical substances in the emissions to waste water at the pulp plant (measurement specifications, see Appendix E “Measurement of emissions to waste water in the production of the pulp”):

• Chemical oxygen demand (COD) in kg O per air dry tonne

• Proportion of chemically oxidising organic compounds in the waste water (usually based on analyses using dichromate oxidation), given as O

• Total nitrogen content in kg N per air dry tonne

• Total-N (Total nitrogen, Tot-N), given as N. This includes organic nitrogen, free ammonia and ammonium (NH4+-N), nitrites (NO2--N) and nitrates (NO3--N).

• Total phosphorous content in kg P per air dry tonne

• Total-P (Tot-P), given as P. This includes both dissolved phosphorous and also undissolved phosphorous which enters the waste water in the form of precipitates or microorganisms.

The following reference values apply to the named substances:

• Chemical oxygen demand: 		CODReference 	= 18.00 kg O/air dry tonne

• Total nitrogen content:		NReference 	= 0.25 kg N/air dry tonne

• Total phosphorous content:		PReference 	= 0.030 kg P/air dry tonne

Based on the measurement values, the applicant must calculate so-called emission points (P) for each of the measured substances as a ratio between the measurement value and the reference value as follows:

The following requirements apply:

For each of the emission points P_COD, P_N and P_P, a value of 1.5 must not be exceed in each case and the sum of the emission points for the emissions to waste water (P_COD, P_N, and P_P) must not exceed a value of 3.0.

3.18.2 Emissions to air in the production of the pulp

The emissions to air include those from the recovery boiler, lime kiln, steam boiler and incinerator for strong smelling gases. Diffuse emissions must also be taken into account. The applicant must determine the levels of the following chemical substances in the emissions to air at the pulp plant (measurement specifications, see Appendix F “Measurement of emissions to air in the production of the pulp”):

• Gaseous sulphur compounds (sulphur) in kg S per air dry tonne

• Total reduced sulphur (TRS): Sum of the following reduced bad-smelling sulphur compounds released during the pulp production process: Hydrogen sulphide, methyl mercaptan, dimethyl sulphide and dimethyl disulfide, given as S, plus sulphur dioxide (SO2), given as S

• Nitrogen oxide (NOx) in Kg NOx per air dry tonne

• Sum of nitrogen oxide (NO) and nitrogen dioxide (NO2), given as NO2

• Dust emissions (dust) in kg dust per air dry tonne

• Sum of the dust emissions at the recovery boiler and lime kiln, given as dust Solid particles of any form, structure or thickness that are dispersed during the gas phase and remain upstream of a defined filter after drying under specified conditions (according to DIN EN 13284-1).

The following reference values apply to the named substances:

• Gaseous sulphur compounds: 	SulphurReference 	= 0.6 kg S/air dry tonne

• Nitrogen oxide: 			NOxReference 	= 1.5 kg NO/air dry tonne

Based on the measurement values, the applicant must calculate so-called emission points (P) for each of the measured substances as a ratio between the measurement value and the refer ence value as follows:

The following requirements apply:

For each of the emission points PSulphur and PNOx, a value of 1.5 must not be exceed in each case and the sum of the emission points for the emissions to air (PSulphur and PNOx) must not exceed a value of 2.0.

Dust emissions must not exceed the limit value of 0.35 kg dust/air dry tonne.

3.18.3 Energy consumption in the pulp production process

The specific energy consumption in the pulp production process must not exceed the following limit values:

• Electrical energy: 	≤1,000 kWh/air dry tonne

• Heating energy: 	≤4,000 kWh/air dry tonne

- Electrical energy (electricity):

The electricity consumed in the production of the pulp must be measured over a period of 12 months and stated in relation to the pulp produced (air dry tonnes) during this period.

The electricity consumption is calculated as follows:

• Electricity consumption = electricity generated at the plant

• plus the electricity purchased from outside of the plant

• less the electricity sold outside of the plant

• less the electricity consumed at the treatment plant

• less the electricity consumed for processes not related to the pulp production at the plant

- Heating energy (fuel):

The heating energy consumed in the production of the pulp must be measured over a period of 12 months and stated in relation to the pulp produced (air dry tonnes) during this period. Heating energy can be in the form of gaseous, liquid or solid fuels (e.g. natural gas, heating oil, biomass) or in the form of heat transfer media (e.g. water, steam). For the energy content of the fuel, the lower heating value (LHV) for the relevant fuel is used. In the case of damp fuels (e.g. wood, biomass), the effective calorific value (after subtracting the evaporation energy of the enclosed water) is used, while the effective energy content is used for heat transfer media.

The heating energy consumption is calculated as follows:

• Heating energy consumption = fuel produced at the plant

• plus the purchased heating energy or fuel

• less the heating energy or fuel sold

• less 1.25 x the electricity generated at the plant

• less heating energy consumed for processes not related to the pulp production at the plant

Note:

The heating energy includes all fuels used (their lower heat value) and the heating energy recovered from the incineration of pulping liquors and waste at the production site (e.g. waste wood, sawdust, pulping liquor, waste paper, rejected paper), as well as the heating energy recovered from the plant’s own electricity generation. The applicant must present the calculation for the energy consumption in the pulp production process in the form of an energy statement together with the calculation parameters used. If the applicant does not have their own heat values for the fuels used, the heat values documented in the Nordic ecolabel for paper products (Nordic Swan Ecolabel) may be used.