|  | Application form (Annex 1) |  |  |
| --- | --- | --- | --- |
|  | DE-UZ 14a – Edition January 2026 | Graphic paper and cardboard made from 100% recovered paper (recycled paper and cardboard) |  |

Company information

| Company name: |  |
| --- | --- |
| Full address: |  |

Contact person

| Name: |  |
| --- | --- |
| Function: |  |
| Phone number: |  |
| E-Mail address: |  |

Product details

| Trade name of the product: |  |
| --- | --- |
| Production site: |  |

Mark which category the product belongs to:

|  | Recycled paper for the production of graphic paper according to the grade statistics for “Graphic Paper” from the German Paper Industry Association (DIE PAPIERINDUSSTRIE e.V.) (Appendix B to these Basic Award Criteria). |
| --- | --- |

In detail:

|  | Paper for printed matter such as books, newspapers, magazines, brochures, catalogues, prospectuses, posters and billboards |
| --- | --- |
|  | Paper for printed matter primarily for use in offices and schools such as exercise books, envelopes and notebooks as well as gift paper and bags |
|  | Office paper (multifunctional and copying paper) |
|  | Writing paper |
|  | Continuous paper for IT applications |
|  | Others (acc. to appendix B): |

|  | Selected types of paper and paperboard for technical and special applications |
| --- | --- |

| We hereby declare compliance with all applicable requirements according to Section 3 of the basic award criteria. |  |
| --- | --- |

3.1 Use of fibrous raw materials and grades of recovered paper

The paper fibres in the recycled graphic paper must have been sourced 100% from post-consumer recovered paper.

For the production of the product named in the application, at least 50% of the recovered paper – based on the total content of fibrous raw materials – must be sourced from the ordinary, medium and kraft paper grades and special grades (groups 1, 2, 4 and 5).

A maximum of 50% of the recovered paper may be sourced from group 3, of which only 70% may be uncoated and adhesive-free recovered paper from the grades listed in Appendix G (i.e. only 35% of the total recovered paper used in the product).

As an exception to this requirement, all paper designed for contact with foodstuffs or which will foreseeably come into contact with foodstuffs must be produced from 100% recovered paper, of which at least 50% must be sourced from the ordinary, medium and kraft paper grades and special grades (groups 1, 2, 4 and 5).

If the grades 2.05.00 ordinary sorted office paper, 2.05.01 sorted office paper, 2.06.00 ordinary sorted coloured letters, 2.06.01 sorted coloured letters and 5.09.00 carbonless copy paper (NCR) are used, the product must comply with the requirement according to Paragraph 3.2 (DIPN content).

Refer to DIN EN 643 for specifications about the different grades of recovered paper.

3.2 Diisopropylnaphthalene (DIPN)

The content of diisopropylnaphthalene (DIPN) in paper and cardboard should be kept as low as technically possible. It is thus generally not permitted to use the grades of recovered paper 2.05.00 ordinary sorted office paper, 2.05.01 sorted office paper, 2.06.00 ordinary sorted coloured letters, 2.06.01 sorted coloured letters and 5.09.00 “carbonless copy paper (NCR)”. Alternatively, grades of recovered paper containing DIPN (2.05.00, 2.05.01, 2.06.00, 2.06.01 and 5.09.00) may be used if an efficient technical system (e.g. deinking) exists that largely removes the DIPN from the fibre cycle and the DIPN content in the finished paper does not exceed a maximum of 50 mg/kg.

3.3 Colour developers from thermal paper

The use of recovered paper could transfer colour developers from thermal paper to the finished paper. Therefore, the content of bisphenol A (BPA), bisphenol S (BOS) and N-(p-toluolsulfonyl) N'-(3-(p-toluolsulfonyloxy)phenyl)urea3 in the finished paper must be determined in a cold water extract once a year.

Depending on the type of product, the content of BPA (CAS no. 80-05-7), BPS (CAS no. 80-09 1) and N-(p-toluolsulfonyl)-N'-(3-(p-toluolsulfonyloxy)phenyl)urea (CAS no. 232938-43-1) must be determined in a cold water extract prepared according to DIN EN 645 using liquid chromatography and UV/fluorescence detection or MS detection in accordance with CEN/TS 17497.

3.4 General exclusion of substances with certain properties

No substances or mixtures may be added as colourants, coating materials, production aids and paper refining agents that contain constituent components with the following properties:

- It is prohibited to add substances of very high concern (SVHC) that have been identified as being particularly alarming in accordance with Article 57, Paragraph 1 of Regulation (EC) No 1907/2006 (REACH) and added to the so-called “candidate list” according to Article 59, Paragraph 1 of the same regulation.

- No substances may be added to the product that

- according to the criteria of Regulation (EC) No 1272/2008 (CLP) are classified with the following H Phrases named in Table 1 or which meet the criteria for such classification,

- or which are classified as carcinogenic, mutagenic or reprotoxic substances in the currently valid version of TRGS 905.

Table 1: H Phrases according to the CLP Regulation

| H Phrases according to the CLP Regulation | Wording |
| --- | --- |
| H340 | May cause genetic defects. |
| H341 | Suspected of causing genetic defects. |
| H350 | May cause cancer. |
| H350i | May cause cancer if inhaled. |
| H351 | Suspected of causing cancer. |
| H360F | May damage fertility. |
| H360D | May damage the unborn child. |
| H360FD | May damage fertility. May damage the unborn child. |
| H360Fd | May damage fertility. Suspected of damaging the unborn child. |
| H360Df | May damage the unborn child. Suspected of damaging fertility. |
| H361f | Suspected of damaging fertility. |
| H361d | Suspected of damaging the unborn child. |
| H361fd | Suspected of damaging fertility. Suspected of damaging the unborn child. |

3.5 Further requirements for production aids and paper refining agents, exclusion of glyoxal

Only those production aids and paper refining agents that are listed in Recommendation XXXVI from the BfR for “Paper and board for food contact” (positive list) in sections B (production aids) and C (special paper refining agents) may be added to the product. The maximum quantities and concentrations stated in this list must be observed.

No production aids containing glyoxal may be used to manufacture the recycled paper.

3.6 Bleaching and complexing agents

The recovered paper must be processed without the use of chlorine, halogenated bleaching agents and not readily biodegradable complexing agents such as e.g. ehylenediaminetetraacetic acid (EDTA) and diethylenetriaminepentaacetic acid (DTPA).

3.7 Biocides

In the production of the recycled paper, only those slimicides (substances in product type 12) and preservatives for fibres (substances in product type 9) that have been approved in accord ance with the Biocidal Products Regulation (EU) No. 528/2012 (EU list of approved active sub stances) or are still being examined as a notified existing active substance for the relevant type of biocides as part of the EU work programme for the systematic examination of all existing active substances may be used.

Accordingly, it is only permitted to use those biocidal products classified in product types 9 and 12 that have been explicitly approved for the desired application. For a transitional period, biocidal products that contain notified existing active substances of product types 9 and 12 that are still being examined as part of the EU examination process can also be used without approval if they have been registered in accordance with the German ordi nance on the notification of biocidal products pursuant to the German Chemicals Act (Biocide Notification Ordinance – ChemBiozidMeldeV) and can thus be made available on the market. Until the approval requirements for the biocidal products containing notified existing active sub stances come into force, however, only those substances that are also listed in Recommendation XXXVI from the BfR are permitted.

In addition, the biocidal products used in the product must not contain any substances that have been considered as candidates for substitution according to Article 10 of the EU Biocidal Products Regulation 528/2012.

It is also possible that production aids and paper refining agents used for the production of the recycled paper contain biocidal products in product type 6 (preservatives for products during storage) that have been made available on the market. Residual content of these biocidal prod ucts will be accepted.

3.8 Whiteness

It is not permitted for any of the products to exceed a maximum grade of whiteness of 100% (including the UV proportion) according to ISO 2470 and a maximum CIE whiteness of 135 according to DIN ISO 11475. Coloured paper is excluded from this requirement.

3.9 Optical brighteners

The use of optical brighteners is not permitted.

An exception applies for the production of:

• SC papers > 110 g/m2 and

• HWC papers > 75g/m2 (according to Appendix B)

The following optical brighteners may be added to these products:

- C.I.220, benzenesulfonic acid, 2,2'-(1,2-ethenediyl) bis[5[4-[bis(2-hydroxyethyl) amino]-6-[(4-sulfophenyl)amino]- 1,3,5-triazin-2-yl]amino]-, tetra sodium salt (CAS no. 16470-24-9);

- C.I. 113 or C.I. 28 disodium salt 4,4'-bis[6-anilino-4-[bis(2-hydroxyethyl)amino]-1,3,5-tri-azin-2-yl]amino]stilbene-2,2'-disulphonate; sulfonated stilbene derivatives may be used up to a maximum level of 0.3%;

- Tetrasodium 4,4'-{ethene-1,2-diylbis[(3-sulfonato-4,1-phenylene)imino{6-[bis(2-hydroxy-ethyl)amino]-1,3,5-triazine-4,2-diyl}imino]}dibenzoate (CAS RN 32257-57-1) and isomeric mixtures of Tetrasodium 4,4'-{ethene-1,2-diylbis[(3-sulfonato-4,1-phenylene)-imino{6-[bis(2-hydroxyethyl)amino]-1,3,5-triazine-4,2-diyl}imino]}dibenzoate (CAS-RN 32257-57-1), Tetrasodium 2,2'-{ethene-1,2-diylbis[(3-sulfonatobenzene-4,1-diyl)imino{6-[bis(2-hy-droxyethyl)amino]-1,3,5-triazine-4,2-diyl}imino]}dibenzoate (CAS RN 158256-89-4) and Tetrasodium 2-({4-[bis(2-hydroxyethyl)amino]-6-[(4-{2-[4-({4-[bis(2-hydroxyethyl)-amino]-6-[(4-carboxylatophenyl)amino]-1,3,5-triazin-2-yl}amino)-2-sulfonatophenyl]-eth-enyl}-3-sulfonatophenyl)amino]-1,3,5-triazin-2-yl}amino)benzoate (CAS no. 1271742-13-2);

- C.I.397 (benzenesulfonic acid, 2,2‘-(1,2-ethenediyl)bis[5-amino-, reaction products with aniline, diethanolamine, ethanolamine and 2,4,6-trichloro-1,3,5-triazine, sodium salts, 2-(Dimethylamino) ethanol compounds (CAS no. 1627851-12-0).

3.10 Azo dyes and pigments in colourants

No azo dyes or pigments may be added in colourants that can cleave into one of the amines stated in Regulation (EC) No. 1907/2006, Annex XVII, No. 43, Appendix 8 or 9, or in TRGS 614 (see Appendix D).

3.11 Mercury, lead, cadmium or chromium VI compounds in colourants

It is not permitted to add any colourants (pigments or dyes) containing mercury, lead, cadmium or chromium (VI) compounds as constituent ingredients.

3.12 Substitution of mineral oil-based additives and base oils

No mineral oil-based additives or base oils that contain aromatic hydrocarbons (with ≥ 10 carbon atoms) as a component may be added during the production of the recycled paper.

In the case of aliphatic hydrocarbons, only those substances with a chain length of C10 to C20 may be used as constituent components. In addition, the following high-molecular compounds without solvent properties may be used if they have a carbon number C > 35 and the proportion of those with a carbon number of C20 to C35 does not exceed a maximum of 5%: microcrystal line waxes, Vaseline, polyolefin waxes, paraffin waxes or Fischer-Tropsch waxes. Plant-based base oils designed for use as a foodstuff or as a material that comes into contact with foodstuffs are exempt from this requirement. Plant-based substitutes for mineral oil should be free of ge netic engineering and sourced from sustainable cultivation.

3.13 Emissions of volatile organic compounds in copying, multifunctional and digital printing paper

Recycled paper designed for use with electophotographic printers or copiers (office paper/digital printing paper) must be tested for its emission potential for the emission of volatile organic compounds (TVOC and TSVOC and DIPN).

The test must be carried out using thermal extraction (TE) on a batch of the packaged paper in accordance with the test guidelines in Appendix C to the Basic Award Criteria DE-UZ 14a. The TE values determined during the test indicate the emission potential and must not exceed the following values:

TVOC: 60 micrograms per gram of paper (μg/g)

TSVOC: 180 micrograms per gram of paper (μg/g)

DIPN: 20 micrograms per gram of paper (μg/g)

- 3.14 Waste water

The following requirements apply with respect to waste water:

3.14.1 Direct discharge

In the production of recycled paper, the emission limits for the waste water load that are listed by the EU Commission in the “Best Available Techniques (BAT) Reference Document for the Production of Pulp, Paper and Board PP BREF“ must be complied with by direct dischargers. These limits have been implemented in Germany in Appendix 28 of the German Waste Water Ordinance. The values stated in Table 2 go above and beyond the statutory requirements in some cases.

Table 2: Maximum limits for the average annual emission parameters (waste water) in the paper production process

| Parameter | Maximum limit for waste water emissions (average annual value as a load or concentration) |
| --- | --- |
| Volumetric flow rate of waste water | 15 m3/Adt |
| COD | 3 kg/Adt |
| BSB5 | 0,15 kg/Adt oder 25 mg/l |
| AOX | < 0,01 kg/Adt |
| Total N (inorganic + organic N) (TNb) | 0,07 kg/Adt oder 15 mg/l |
| Total P | 0,008 kg/Adt oder 1,2 mg/l |

Adt = air dried ton

TNb = total nitrogen bound. This parameter defines the total pollution of water by nitrogen compounds, which can appear in the form of e.g. ammonia, nitrites, nitrates or organic nitrogen compounds. A suitable method for determining this parameter is DIN EN ISO 20236.

3.14.2 Indirect discharge

Indirect dischargers must declare compliance with the emission limits for the parameters stated in Table 2 after treatment.

3.15 Emissions to air

Requirements are placed on the emissions to air in the production of recycled paper. The emissions to air include both the emissions from the plants generating the steam needed for the production of the paper and also the emissions from the plants used to prepare the recovered paper as well as the paper machine. Emissions during production of the fibrous raw materials (DIP) also have to be taken into account. The limits listed here in Table 3 are based on the EU criteria for the award of the EU Ecolabel for graphic paper (Annex I) in the Official Journal of the European Union of 17 January 2019.

The applicant must determine the levels of the following pollutants in the emissions to air at the paper factory and should comply with the limits stated in Table 3 (measurement specifications, see Appendix E “Measurement of emissions to air”):

Table 3: Maximum limits for the average annual emission parameters (emissions to air) in the paper production process (average annual value (AAV) in kg/air dry tonne)

|  | Sulphur (S) as AAV | NOx as AAV |
| --- | --- | --- |
| Preparation of the recovered paper | 0,20 kg/t | 0,25 kg/t |
| Production of recycled paper | 0,30 kg/t | 0,50 kg/t |

3.16 Waste

To promote the avoidance of waste and in the spirit of a circular economy, the waste generated during the production process must be minimised. The following waste materials are generated during the production of paper from recovered paper: Sludge from the treatment of the process water, fibrous sludge, residues from the treatment of the recovered paper and deinking sludge. The remaining waste materials should be recycled using the highest possible recycling standards.

In the production of recycled paper, the following waste material limits, including sludge from the treatment of the process water, given as an annual average figure as a dry mass, must not be exceeded:

Paper factory with deinking: 250 kg/t of product

Paper factory without deinking: 135 kg/t of product

These limits were derived from the data collected for the BAT reference document for the paper and pulp industry (PP BREF, Section 6.2.1., Table 6.1).

3.17 Energy consumption and origin of the electricity

The paper industry is one of the six most energy-intensive industries in Germany. The Blue Angel also sets incentives in its Basic Award Criteria for companies to further reduce the consumption of heat and electricity at the production sites. For the production of recycled paper, the limits for the consumption of electricity and process heat in the following table must not be exceeded as an annual average:

If deinked recovered paper (DIP) is purchased as a semi-finished product, the energy consumption for additional drying and transport processes must also be taken into account. The sum of the process heat and electrical power is permitted to exceed the limit by a total of 10%.

In addition, the applicant must state the energy mix used at the plant based on the type and origin of the energy. The consumed electricity should be sourced from renewable energies or residual materials. The applicant must also state when he or she will stop using coal as an energy source, if this has not already occurred.

Table 4: Maximum limits for the consumption of process heat and electricity in the production of paper (annual average value in kWh/t)

|  | Process heat (including power-to-heat) in kWh/t | Electrical energy (excluding power-to-heat) in kWh/t |
| --- | --- | --- |
| Paper factory with deinking | 1.550 | 900 |
| Paper factory without deinking | 1.550 | 600 |

These limits were derived from the data collected for the BAT reference document for the paper and pulp industry (PP BREF, Section 6.2.1, Table 6.7) and for PTS Munich: Report No. 2 Use of energy saving techniques, for the revision of the BAT reference document for the pulp and paper industry 2009, Table 14).

3.18 Fitness for use

The fitness for use of the recycled paper must be guaranteed. If relevant DIN standards include technical requirements for individual products, these must be observed in the versions valid at the time of application. This applies to e.g.

• Continuous paper: DIN EN 12858

• Envelope paper: DIN 6733

• Paper and board for office purposes: DIN 19307

• Paper for copying purposes: DIN EN 12281