|  | Leaflet – Circular economy concept according to 3.4 |  |  |
| --- | --- | --- | --- |
|  | DE-UZ 132 - Edition July 2026 | Low-Emission Thermal Insulation Products and Suspended Ceilings for Indoor Use |  |

- The circular economy concept comprises the following points:

- dismantling instructions are to be provided in Germany and in other countries in which the product is marketed, which describe for those carrying out the work how to handle the product in the event that constructions typical of the product are dismantled in the future. It should be explained how the separation of the product by material type (> 95%) can be achieved using the current state of the art.

- A concept is to be provided for the product end-of-life, which

  - describes the anticipated take-back channels (e.g. regional collection centres, decentralised collection systems, cooperation with existing take-back systems or specialised third-party providers) for the return of construction site offcuts and demolition material from the product by material type

  - states at least one reuse or recycling procedure for all constituents of the product according to the current state of the art and provides information about current recovery channels.

- guaranteed take-back of the used product and coverage of costs for at least 50 years provided that the product was build back according to the dismantling instructions. Customers may only be required to contribute to the logistics costs of the take-back channel to the extent that would be customary on the market for the delivery of the product at the time of its return.

| Requirement | Scope |
| --- | --- |
| Dismantling instructions | Required: - specification of the fixings and their arrangement - specification of tools available today that are suited for the use at construction sites - procedure of loosening the fixings - specification of the material fractions after full dismantling - health and safety guidelines - readable note on the dismantling instructions for enclosing to the building documentation that it remains available at the product’s end of life Optional: - if necessary, technical drawings - if necessary, images of the surface quality after separation - permanent link to the MSDS - With more practical instructions, an end-of-life material of higher quality is expected. |
| Material specific separation | The dismantling instructions shall describe the tools available today and methods that allow the separation of the material fractions from the composite material so that no material fraction contains more than 5% of adhering contaminants hindering a circular use. This applies to the material fractions of the product as well as the intended fixings with the building. |
| Take-back channels | The manufacturer shall describe how the take-back systems for the construction cut off and demolition material is organised. There are no requirements. A takeback is allowed for reuse in the own production process or in cooperation with other manufacturers. It is recommended to plan for the highest possible future value creation as the manufacturer covers the cost for the chosen take-back channel. The manufacturer shall state contact details for the take-back of used material that will be reachable over the guaranteed period of 50 years. If applicable, additional contact details of a third party can be stated for the handling of the take-back system. |
| Reuse/Recycling | Customers shall be informed about today’s possibilities for re-use with retaining the product appearance or reutilisation with dissolution of the product appearance (e.g. recycling) and which of the possibilities is the present main recovery route of the product. A differentiation between construction cut offs and end-of-life material is allowed, e.g. at the time of publication of this document, it is possible to physically recycle the product without any loss of quality. At present, once it has reached the end of its life, it is mainly sent to waste incineration plants and is thus removed from the material cycle. |
| Guaranteed take-back | Customers shall be informed that the product can be returned to the manufacturer free of charge if the product is dismantled and separated according to the dismantling instructions. The customers shall be informed that they needed to contribute towards the cost up to the standard delivery charges applicable at the time of return. The manufacturer is only allowed to request information and proofs at the time of return that were handed over at the time of purchase with the information documents containing the guaranteed take-back. |